To provide a robust power solution for the AgroSmart Hub, we must account for both the "Muscle" (Motors) and the "Brain" (Arduino and Sensors), they contribute to a constant parasitic draw that must be factored into the 1-hour runtime.

1. Total Load Power 	(P = V x I)

| System Component | Qty | Voltage | Current (a) | Power (Watts) | Wh (1 hr) |
| --- | --- | --- | --- | --- | --- |
| TT Gear Motor | 3 | 6V | 1.5A | 27.0 W | 27.0 Wh |
| Servo Motor 1 | 1 | 5V | 0.5A | 2.5 W | 2.5 Wh |
| Servo Motor 2 | 1 | 6V | 0.5A | 3.0 W | 3.0 Wh |
| DC Motor | 2 | 6V | 1.5A | 18.0 W | 18.0 Wh |
| Arduino + 3 Sensors | 1 | 5V | ~0.5A* | 2.5 W | 2.5 Wh |
| Base Load Total |  |  |  | 53.0 W | 53.0 Wh |
| 53.0 Wh Becomes the battery energy requirement to run the hub for 1 hour |  |  |  |  |  |
| 2. Battery Sizing (Needed Battery Energy) The energy requirement to run a load of 53W for one hour is 53Wh. This becomes the base battery energy. Since we are not connecting the battery to the loads directly, rather we are connecting the battery to other components like BMS, Buck converter etc. these components will make the battery lose some energy. If you want your load to still run for 1 hour as calculated, you need to add those loses to the total Load energy estimate. Lets find those loses. Factoring in Losses |  |  |  |  |  |
| BMS efficiency is 97% (meaning 3% energy loss) | E = output power /input 97% = 53/in Input = 53/0.97 = 54.64 |  |  |  |  |
| This means that while using BMS, we need a battery energy of 54.64Wh to run a load of 53Wh for 1 hour (Energy Loss = 54.64 – 53 = 1.64Wh) |  |  |  |  |  |
| Bulk converter efficiency is 85% (meaning 15% energy loss) | E = output power /input 85% = 54.64/in Input = 54.64/0.85 = 64.28 |  |  |  |  |
| This means that while using BMS, and Buck converter, we need a battery energy of 64.28Wh to run a load of 53Wh for 1 hour (Energy Loss = 64.28 – 53 = 11.28Wh) |  |  |  |  |  |
| 20 % Engineering Power Loss meaning 80% efficiency | E = output power /input 80% = 64.28/in Input = 64.28/0.80 = 80.35 |  |  |  |  |
| This means that while using BMS, and Buck converter, and engineering setup for the project we need a battery energy of 80.35Wh to run a load of 53W for 1 hour (Energy Loss = 80.35– 64.28 = 16.07Wh) |  |  |  |  |  |
| Efficiency of Battery (DoD) is 75% (meaning 25% energy would not be used) | E = output power /input 75% = 80.35/in Input = 80.35/0.75 = 107.13Wh |  |  | 107.13Wh |  |
| This means that while using BMS, Buck converter, Load and the Battery itself, we need a battery energy of 107.13Wh to run a load of 53Wh for 1 hour (Energy Loss = 107.13 - 80.35= 26.78Wh) |  |  |  |  |  |
| Final Battery Energy Requirement to run this 53W load for 1 hour | 107.13Wh |  |  |  |  |

3. Lithium-ion Battery Configuration

To determine the battery configuration that will supply 107.13Wh of energy, we need to first consider the highest voltage in the component stack.

We have these voltages in the component stack 6V component and 5V component, and 12V component, thus the highest voltage is 12V.

To power a 12V component, we need a voltage that has a higher overhead, something between from 12.6 – 14v (I will use 13v).

We can now use this 13V as the base voltage our series connection should meet. To get the number of series that will be in the pack, divide 13 by 3.7 i.e. (13/3.7), that is 3.5, approx. 4 cells in series. A 4S (4 series) pack is therefore ideal to provide the voltage needed by our 12V component. A 4S (4series) pack provide 14.8v nominal voltage and 16.8v ma voltage. We will use a DC-DC buck converter to step it down to the voltage requirement of each load. 4S series have over 2.8V over head.

Now we calculate for the Capacity of the battery needed

To get the capacity of the setup, we divided the final load energy requirement (107.13WH) by the total voltage of the series (because energy is V x capacity Ah), this gives 107.13/14.8 = 7.2 Approx. 7Ah

To get the number of cells that will be in parallel, we divide 7 by the capacity of one cell (1.5A), that should give us 4.82 cells, approx. 5 cells in parallel.

- Configuration: 4S5P (4 Cells in Series, 5 Cells in Parallel)

- Total Cells: 20 Cells (3.7V, 1.5Ah each)

- Battery Voltage: 14.8V (Nominal) / 16.8V (Full Charge)

- Pack Capacity: 7.5Ah (1.5 x 5) Approx. 8Ah

- Total Energy Capacity: 111Wh (14.8V x 7.5 Ah = 111Wh)

Energy demand for our system is 107.13Wh, but we will have a 111Wh battery pack from a 4S5P pack (3.7V 1.5Ah). This is more than what our system needs, and it is good.

4. Solar Panel Capacity

To replenish the 107.13Wh consumed during the 1-hour demonstration within a standard 4-hour peak sunlight window:

Required Panel Power

Panel Power = Energy ÷ (Sun hours × panel Efficiency)

To build a system that will power the load whilst still charging the battery at the same time, then we must add the load energy requirement with the battery energy to get your total energy requirement.
Load energy (with loses) = 80.35Wh for 1 hour (Note that I removed 75% Depth of discharge since the load is powered from panel)

Battery energy = 111Wh

Total energy required for the panel to supply = 80.35 + 111 = 191.35Wh

Panel Power = Total Energy ÷ (Sun hours × panel Efficiency)

Using 4 hours (although peak day sunlight is 5h, just to assume the least sun available):

191.35 ÷ (4 × 80%) = 59.8W ≈ 60W

So to be able to run our 53W load for 1 hour while making our battery setup charge to full (witing a 4 hours peak sunlight), we need a Approx. 60W solar panel

To get the voltage the solar panel should have, consider the max voltage of the battery pack (because the panel connects to the battery to charge it), which is 16.8V, so we will choose at least an 18V solar panel (with 3.33A). I = 60/18 = 3.33A

Or we choose a 19V solar panel (with 3.16A). I = 60/19 = 3.16A

I bought an 80W Monocrystalline Solar Panel. This provides enough power to charge the battery while the loads are running in standby mode during the day.

5. BMS Sizing

To determine the ideal BMS for your 4S5P pack, we first need to translate the "Wh" load into "Amps," as BMS units are rated by current (Amps), not energy (Wh).

1. Calculating The Current Draw

Since we are using a 4S (4 cells in series) Li-ion pack, your nominal voltage is 14.8V (3.7V per cell).

- Load Power: 80.35W (assuming the 80.35Wh load is consumed over 1 hour)

- Formula: Amps (A) = Watts (W) / Volts (V)

- Calculation: 80.35W / 14.8V approx. 5.43 A

The BMS device will pull approximately 5.43 Amps continuously.

2. The Ideal BMS Rating

To ensure the BMS doesn't overheat or "trip" during a minor surge, I will apply a 30% to 50% safety buffer.

At 30%:
	0.3 x 5.43 = 1.63 A

Add it as a buffer to the initial current calculated: 1.63 + 5.42 = 7.05 Amps

At 50%:
	0.5 x 5.42 = 2.71 A

Add it as a buffer to the initial current calculated: 2.71 + 5.42 = 8.13 Amps

- Minimum Rating: 10 Amps -12 Amps

- Ideal Rating: 15A to 20A

A 15A or 20A BMS is the "sweet spot" for your setup. It is large enough to handle your 60W load with ease but small enough to remain cost-effective and compact.

6. Charge Controller Sizing

Determining Your Pack Capacity

First, I convert the energy (111Wh) into Amp-hours (Ah) to see how much "room" is in the tank.

- Total Energy: 111Wh

- Nominal Voltage (4S): 14.8V

- Capacity: 111Wh / 14.8V = 7.5Ah (or 7500mAh)

2. Then I will Choose the controller Amperage

For Lithium-ion, the standard charging rate is usually between 0.2C (gentle) and 0.5C (fast).

- The "Gentle" Rate (0.2C): 7.5 Ah x 0.2 = 1.5A

  - Best for: Maximizing the lifespan of your cells. The battery will stay cool.

- The "Standard" Rate (0.3C): 7.5Ah x 0.3 = 2.25A

  - Best for: A great balance between speed and battery health.

- The "Fast" Rate (0.5C): 7.5Ah 0.5 = 3.75A

Choosing the Charging Rate (C-Rating)

For Lithium-ion health, I aim for a balance between speed and cell longevity.

- The "Safe" Rate (0.25C): 7.5Ah x 0.25 = 1.9A.

- Why this is perfect: The 80W Solar Panel at 19V naturally outputs approximately 4.21A 80W/ 19V.

4.21A does not match my gentle 1.9A–2.25A target. In fact, 4.21A is higher than the maximum "Fast Rate" limit (3.75A).

- To get the absolute best of both worlds—the perfect 0.3C (2.25A) gentle charge current, Or 0.5C (3.75A) Fast Rate Current and the reliable 16.8V automatic safety cutoff—I would use a programmable (can be set) 10A Solar Charge Controller (either PWM or MPPT) that explicitly allows me to adjust the "Maximum Charging Current" parameter in its settings

Or

I would Calibrate an XL4016 Buck converter.

How to calibrate your XL4016 for your 4S pack:

The XL4016 is a CC/CV (Constant Current / Constant Voltage) module, it has two adjustable screws, one for controlling current and one for controlling voltage.

To run the load while charging, I first set these limits using a DC adapter (19v, 3 Amps – 5 Amps) connected to my wall socket before I connect the battery.

- Set Voltage (CV):

  - Connect your 19V DC wall adapter to the IN+ / IN- terminals for calibration.

  - With the output wires disconnected, use your multimeter to measure the voltage at the OUT+ / OUT- terminals.

  - Ordinarily, I should turn the voltage potentiometer until the display reads 16.8V (which is the 4S full-charge limit). Because the Schottky diode causes a ~0.4V drop and the fuse adds a tiny resistance, I will adjust the voltage potentiometer until my multimeter reads exactly 16.8V at the very end of the charging line (which means the raw XL4016 output will naturally sit around 17.2V–17.3V). This ensures exactly 16.8V reaches the battery.

- Set Current (CC):

  - Switch your multimeter to the 10A DC (or 20A DC) setting and move the red probe to the 10A (or 20A) port.

  - Touch the probes directly to the OUT+ and OUT- terminals (this creates a temporary short circuit).

  - Turn the current potentiometer until the meter reads 3.0A to 4.0A. This ensures the XL4016 won't pull too much current from the solar panel and won't charge your battery too fast.

Note: If using the short-circuit method, ensure the current pot is turned mostly counter-clockwise before touching the probes to avoid a sudden high-current spark that could blow your multimeter's internal fuse.

  - OR to calibrate the current limit safely without a short circuit, you must connect a known physical load (like a heavy-duty ceramic power resistor or an automotive halogen bulb) in series with your multimeter. Let the system run safely through the load, read the current on your meter, and turn the CC pot down until the current drops and locks onto your target 2.25A (or your chosen 3.0A limit).

- The "Night Flow" Warning:

- The Issue: At night, when the sun goes down, the XL4016 will remain "on" by pulling power backwards from my battery. This will slowly drain my 4S pack and could damage the XL4016 chip over time.

- The Solution: I would install a blocking diode (like a 10A Schottky diode) on the positive output wire of the XL4016 before it reaches the battery. I would connect a 5 Amps fuse directly to the output of XL4016, then connect the Schottky diode to the other end of the fuse

- Diode to pick for blocking current from flowing:
10A Schottky diode 10SQ045 or 10SQ050

- Note:

The XL4016 is a Buck Converter, Unlike the MPPT controller:

- It only works if the Solar Panel Voltage (18V-21V) is higher than the Battery Voltage (16.8V).

- On a very cloudy day, if the panel voltage drops below 17V, the XL4016 will stop charging entirely.